Preliminary information for the planning companies

The following tender text is intended to assist you in drawing up service specifications.

We ask you to clearly mark the selected device for the project and to delete the non-selected device types.

CAD files (dwg, stp format) of the devices are available on request

Flap-type flow indicator for measuring liquids with flange connection DN 15 to DN 150 (½“ to 6“)

Flap-type flow indicator are used in the plant and mechanical engineering, chemical, building, air-conditioning and heating technology sectors, as well as in the water and waste water sector.

Operating conditions

Measurement range:	0,5 to 160 m³/h (depending on the diameter)

Max. ambient temperature: 	+90 °C

Max. operating temperature: 	+100 °C

Max. operating pressure: 	10 bar

Measurement deviation:	±5 % of full-scale deflection

Installation orientation:	horizontal or vertical

Versions

Housing: Grey-cast iron, pressure rings: steel, measuring flap and bearing stainless steel, seal: FKM

Housing: Stainless steel, pressure rings: stainless steel, measuring flap and bearing stainless steel, seal: FKM

Process connection

Flange EN 1092-1: DN 15 to DN 150 / ANSI B16.5: 1“ to 6“

Protection degree:

INTRA: (for measuring transparent liquids): based on IP 54
PRIMA (pointer mechanism, for measuring opaque / cloudy liquids): based on IP 54

Permits and approvals

Classification according to the pressure vessel directive 2014/68/EG 4.3

Measuring range (based on measurement of H2O)

INTRA (visual indicator) / PRIMA (pointer mechanism)

| DN | Measuring range |  | INTRA | PRIMA | build-in length |
| --- | --- | --- | --- | --- | --- |
|  | horizontal (m³/h) | vertical (m³/h) | (bar) | (bar) | (mm) |
| 15 | 0,2 -1,0 0,3 - 6,0 | 0,2 - 1,5 1,0 - 6,5 | 10 | 10 | 170 |
| 20 | 0,2 - 1,0 0,3 - 6,0 | 0,2 - 1,5 1,0 - 6,5 | 10 | 10 | 170 |
| 25 | 0,2 - 1,0 0,3 - 6,0 | 0,2 - 1,5 1,0 - 6,5 | 10 | 10 | 170 |
| 32 | 0,5 - 4,0 3,0 - 30 | 0,5 - 4,0 2,5 - 25 | 9,0 | 10 | 240 |
| 40 | 0,5 - 4,0 3,0 – 30 | 0,5 - 4,0 2,5 - 25 | 9,0 | 10 | 240 |
| 50 | 0,5 - 4,0 3,0 - 30 | 0,5 - 4,0 2,5 - 25 | 9,0 | 10 | 240 |
| 65 | 1,0 - 8,0 2,5 - 50 | 2,0 - 15 5,0 - 40 | 10 | 10 | 280 |
| 80 | 1,0 - 10 7,0 - 70 | 2,0 - 10 5,0 - 50 | 10 | 10 | 320 |
| 100 | 1,5 - 15 12 - 120 | 1,5 - 15 10 - 100 | 10 | 10 | 350 |
| 125 | 2,0 - 20 14 - 140 | 2,0 - 20 12 - 120 | 7,0 | 10 | 380 |
| 150 | 2,0 - 25 16 - 160 15 - 200* | 4,0 - 25 15 - 140 | 6,5 | 10 | 380 |

*only with glass pane (The table indicates the minimal and maximal measuring range. All ranges between are on request.)

Accessory

Exclusively for PRIMA:

1 x Inductive contact - SB3,5-E2 (3-wire system - non Ex)
Temperature resistance: -25 to + 70 °C

1 x NAMUR Inductive contact - SC3,5-N0-BU (2-wire system)
Temperature resistance: -25 to + 100 °C

Features (optional available)

Silicon-free version

ATEX version

Special scaling (double scale) only for PRIMA (flap-type indicator with visual indicator)

Manufacturer: 	MECON GmbH

Type:	INTRA (for measuring transparent liquids) / PRIMA (for measuring
 	opaque / cloudy liquids)

Contact: Michael Juech, +49 (0)22 37 60006-14, m.juech@mecon.de